2190151 Computer Programming Lab

Lab 4: Speaker, Digital clock and Stopwatch

Objectives:

1.Write a program to play a melody on M5Stack built-in Speaker.

2.Write simple program to show the time on TFT LCD screen.

3.Design a program that use TFT and button function.

4. Create a simple watch function using selection and loop.

Task 1: Speaker, play Jingle Bells

Step 1. Go to the folder Arduino-ESP32-IDE and open the app arduino.exe (Fig. 1);

Figure 1. The archive with the Arduino IDE extracted to the folder

Step 2. On the File menu, select New (Fig. 1.1);

Figure 1.1. Create a new sketch

Step 3. Select Include Library, M5Stack in the Sketch menu (Fig. 1.2);

Figure 1.2. Connection library M5Stack

Figure 1.3. The holes for the sound from the built-in speaker are located on the right side of the device

Figure 1.4. Speaker is located on the left of the bus 2 to 15

Step 4. Add to the beginning of system functions void setup() function call M5.begin (); because it is necessary to prepare the device to work. Add to the end system functions void loop() function call M5.update (); is required for correct operation of the device with built-in speaker. This step should be performed always when writing a new sketch (Fig. 2).

Figure 2. An example of the correct preparation of the sketch for work with built-in speaker

Step 5. To work with the speaker (Fig. 1.3, 1.4) use the following functions and constants notes in table 1 and 2 respectively:

List 1. Functions for working with built-in speaker

- M5.Speaker.tone(uint32_t freq); to Generate a beep at a specified frequency (Hz)

- M5.Speaker.mute(); Interrupt generation

List 2. Constant notes (frequencies rounded to whole numbers)

- #define NOTE_D0 -1 silence
THE FIRST OCTAVE

- #define NOTE_B1 262 To (B)

- #define NOTE_Cd1 277 C sharp (C#)

- #define NOTE_D1 294 Re (D)

- #define NOTE_Dd1 311 in D sharp (D#)

- #define NOTE_E1 330 Mi (E)

- #define NOTE_F1 349 FA (F)

- #define NOTE_Fd1 370 f-sharp (F#)

- #define NOTE_G1 392 Sol (G)

- #define NOTE_Gd1 415 g sharp (G#)

- #define NOTE_A1 440 La (A)

- #define NOTE_Ad1 466 La-a sharp (A#)

- #define NOTE_H1 494 si (H)

Step 6. Write a music box (Fig. 2.1, 2.2);

Figure 2.1. Code music box. Page first

Figure 2.2. Code music box. Page two

Step 7. Click the Upload button (Fig. 3) in order to flash the device;

Figure 3. Download the firmware to the device

Step 8. When the device firmware is completed, press the A button and enjoy the melody :)

Task 2  Create a program that allow M5 stack  to be a digital clock using the following code

| /* |
| --- |
|  | An example digital clock using a TFT LCD screen to show the time. |
|  | Demonstrates use of the font printing routines. (Time updates but date does not.) |
|  | It uses the time of compile/upload to set the time |
|  | For a more accurate clock, it would be better to use the RTClib library. |
|  | But this is just a demo... |
|  | Make sure all the display driver and pin connections are correct by |
|  | editing the User_Setup.h file in the TFT_eSPI library folder. |
|  | ######################################################################### |
|  | ###### DON'T FORGET TO UPDATE THE User_Setup.h FILE IN THE LIBRARY ###### |
|  | ######################################################################### |
|  | Based on clock sketch by Gilchrist 6/2/2014 1.0 |
|  | A few colour codes: |
|  | code color |
|  | 0x0000 Black |
|  | 0xFFFF White |
|  | 0xBDF7 Light Gray |
|  | 0x7BEF Dark Gray |
|  | 0xF800 Red |
|  | 0xFFE0 Yellow |
|  | 0xFBE0 Orange |
|  | 0x79E0 Brown |
|  | 0x7E0 Green |
|  | 0x7FF Cyan |
|  | 0x1F Blue |
|  | 0xF81F Pink |
|  | */ |
|  | M5.Lcd.fillScreen(TFT_BLACK); |
|  | M5.Lcd.setTextSize(1); |
|  | M5.Lcd.setTextColor(TFT_YELLOW, TFT_BLACK); |
|  | targetTime = millis() + 1000; |
|  | } |
|  | void loop() { |
|  | if (targetTime < millis()) { |
|  | // Set next update for 1 second later |
|  | targetTime = millis() + 1000; |
|  | // Adjust the time values by adding 1 second |
|  | ss++; // Advance second |
|  | if (ss == 60) { // Check for roll-over |
|  | ss = 0; // Reset seconds to zero |
|  | omm = mm; // Save last minute time for display update |
|  | mm++; // Advance minute |
|  | if (mm > 59) { // Check for roll-over |
|  | mm = 0; |
|  | hh++; // Advance hour |
|  | if (hh > 23) { // Check for 24hr roll-over (could roll-over on 13) |
|  | hh = 0; // 0 for 24 hour clock, set to 1 for 12 hour clock |
|  | } |
|  | } |
|  | } |
|  | // Update digital time |
|  | int xpos = 0; |
|  | int ypos = 85; // Top left corner ot clock text, about half way down |
|  | int ysecs = ypos + 24; |
|  | if (omm != mm) { // Redraw hours and minutes time every minute |
|  | omm = mm; |
|  | // Draw hours and minutes |
|  | if (hh < 10) xpos += M5.Lcd.drawChar('0', xpos, ypos, 8); // Add hours leading zero for 24 hr clock |
|  | xpos += M5.Lcd.drawNumber(hh, xpos, ypos, 8); // Draw hours |
|  | xcolon = xpos; // Save colon coord for later to flash on/off later |
|  | xpos += M5.Lcd.drawChar(':', xpos, ypos - 8, 8); |
|  | if (mm < 10) xpos += M5.Lcd.drawChar('0', xpos, ypos, 8); // Add minutes leading zero |
|  | xpos += M5.Lcd.drawNumber(mm, xpos, ypos, 8); // Draw minutes |
|  | xsecs = xpos; // Sae seconds 'x' position for later display updates |
|  | } |
|  | if (oss != ss) { // Redraw seconds time every second |
|  | oss = ss; |
|  | xpos = xsecs; |
|  | if (ss % 2) { // Flash the colons on/off |
|  | M5.Lcd.setTextColor(0x39C4, TFT_BLACK); // Set colour to grey to dim colon |
|  | M5.Lcd.drawChar(':', xcolon, ypos - 8, 8); // Hour:minute colon |
|  | xpos += M5.Lcd.drawChar(':', xsecs, ysecs, 6); // Seconds colon |
|  | M5.Lcd.setTextColor(TFT_YELLOW, TFT_BLACK); // Set colour back to yellow |
|  | } |
|  | else { |
|  | M5.Lcd.drawChar(':', xcolon, ypos - 8, 8); // Hour:minute colon |
|  | xpos += M5.Lcd.drawChar(':', xsecs, ysecs, 6); // Seconds colon |
|  | } |
|  | //Draw seconds |
|  | if (ss < 10) xpos += M5.Lcd.drawChar('0', xpos, ysecs, 6); // Add leading zero |
|  | M5.Lcd.drawNumber(ss, xpos, ysecs, 6); // Draw seconds |
|  | } |
|  | } |
|  | } |
|  | // Function to extract numbers from compile time string |
|  | static uint8_t conv2d(const char* p) { |
|  | uint8_t v = 0; |
|  | if ('0' <= *p && *p <= '9') |
|  | v = *p - '0'; |
|  | return 10 * v + *++p - '0'; |
|  | } |

| #include <M5Stack.h> |  |
| --- | --- |
| #define TFT_GREY 0x5AEB |  |
| uint32_t targetTime = 0; // for next 1 second timeout |  |
| static uint8_t conv2d(const char* p); // Forward declaration needed for IDE 1.6.x |  |
| uint8_t hh = conv2d(__TIME__), mm = conv2d(__TIME__ + 3); uint8_t ss = conv2d(__TIME__ + 6); // Get H, M, S from compile time |  |
| byte omm = 99, oss = 99; |  |
| byte xcolon = 0, xsecs = 0; |  |
| unsigned int colour = 0; |  |
| void setup() { |  |
| //Serial.begin(115200); |  |
| M5.begin(); |  |
| // M5.Lcd.setRotation(1); |  |
| M5.Lcd.fillScreen(TFT_BLACK); |  |
| M5.Lcd.setTextSize(1); |  |
| M5.Lcd.setTextColor(TFT_YELLOW, TFT_BLACK); |  |
| targetTime = millis() + 1000; |  |
| } |  |
| void loop() { |  |
| if (targetTime < millis()) { |  |
| // Set next update for 1 second later |  |
| targetTime = millis() + 1000; |  |
| // Adjust the time values by adding 1 second |  |
| ss++; // Advance second |  |
| if (ss == 60) { // Check for roll-over |  |
| ss = 0; // Reset seconds to zero |  |
| omm = mm; // Save last minute time for display update |  |
| mm++; // Advance minute |  |
| if (mm > 59) { // Check for roll-over |  |
| mm = 0; |  |
| hh++; // Advance hour |  |
| if (hh > 23) { // Check for 24hr roll-over (could roll-over on13) |  |
| hh = 0; // 0 for 24 hour clock, set to 1 for 12 hour clock |  |
| } |  |
| } |  |
| } |  |
|  | // Update digital time |  |
|  | int xpos = 0; |  |
|  | int ypos = 85; // Top left corner ot clock text, about half way down |  |
|  | int ysecs = ypos + 24; |  |
|  | if (omm != mm) { // Redraw hours and minutes time every minute |  |
|  | omm = mm; |  |
|  | // Draw hours and minutes |  |
|  | if (hh < 10) xpos += M5.Lcd.drawChar('0', xpos, ypos, 8); // Add hours leading zero for 24 hr clock |  |
|  | xpos += M5.Lcd.drawNumber(hh, xpos, ypos, 8); // Draw hours |  |
|  | xcolon = xpos; // Save colon coord for later to flash on/off later |  |
|  | xpos += M5.Lcd.drawChar(':', xpos, ypos - 8, 8); |  |
|  | if (mm < 10) xpos += M5.Lcd.drawChar('0', xpos, ypos, 8); // Add minutes leading zero |  |
|  | xpos += M5.Lcd.drawNumber(mm, xpos, ypos, 8); // Draw minutes |  |
|  | xsecs = xpos; // Sae seconds 'x' position for later display updates |  |
|  | } |  |
|  | if (oss != ss) { // Redraw seconds time every second |  |
|  | oss = ss; |  |
|  | xpos = xsecs; |  |
|  | if (ss % 2) { // Flash the colons on/off |  |
|  | M5.Lcd.setTextColor(0x39C4, TFT_BLACK); // Set colour to grey to dim colon |  |
|  | M5.Lcd.drawChar(':', xcolon, ypos - 8, 8); // Hour:minute colon |  |
|  | xpos += M5.Lcd.drawChar(':', xsecs, ysecs, 6); // Seconds colon |  |
|  | M5.Lcd.setTextColor(TFT_YELLOW, TFT_BLACK); // Set colour back to yellow |  |
|  | } |  |
|  | else { |  |
|  | M5.Lcd.drawChar(':', xcolon, ypos - 8, 8); // Hour:minute colon |  |
|  | xpos += M5.Lcd.drawChar(':', xsecs, ysecs, 6); // Seconds colon |  |
|  | } |  |
|  | //Draw seconds |  |
|  | if (ss < 10) xpos += M5.Lcd.drawChar('0', xpos, ysecs, 6); // Add leading zero |  |
|  | M5.Lcd.drawNumber(ss, xpos, ysecs, 6); // Draw seconds |  |
|  | } |  |
|  | } |  |
|  | } |  |
|  | // Function to extract numbers from compile time string |  |
|  | static uint8_t conv2d(const char* p) { |  |
|  | uint8_t v = 0; |  |
|  | if ('0' <= *p && *p <= '9') |  |
|  | v = *p - '0'; |  |
|  | return 10 * v + *++p - '0'; |  |
|  | } |  |

//Update digital time int xpos = 0;

int ypos = 85; // Top left corner to clock text, about half way down

int ysecs = ypos + 24;

if (omm != mm) { // Redraw hours and minutes time every minute

omm = mm;

// Draw hours and minutes // Add hours leading zero for 24 hr clock

if (hh < 10) xpos += M5.Lcd.drawChar('0', xpos, ypos, 8);

xpos += M5.Lcd.drawNumber(hh, xpos, ypos, 8); // Draw hours

xcolon = xpos; // Save colon coord for later to flash on/off later

xpos += M5.Lcd.drawChar(':', xpos, ypos - 8, 8); // Add minutes leading zero

if (mm < 10) xpos += M5.Lcd.drawChar('0', xpos, ypos, 8);

xpos += M5.Lcd.drawNumber(mm, xpos, ypos, 8); // Draw minutes

xsecs = xpos; // Save seconds 'x' position for later display updates

}

if (oss != ss) { // Redraw seconds time every second

oss = ss;

xpos = xsecs;

if (ss % 2) { // Flash the colons on/off // Set colour to grey to dim colon

M5.Lcd.setTextColor(0x39C4, TFT_BLACK);

M5.Lcd.drawChar(':', xcolon, ypos - 8, 8); // Hour:minute colon

xpos += M5.Lcd.drawChar(':', xsecs, ysecs, 6); // Seconds colon // Set colour back to yellow

M5.Lcd.setTextColor(TFT_YELLOW, TFT_BLACK);

}

else {

M5.Lcd.drawChar(':', xcolon, ypos - 8, 8); // Hour:minute colon

xpos += M5.Lcd.drawChar(':', xsecs, ysecs, 6); // Seconds colon

}

//Draw seconds // Add leading zero

if (ss < 10) xpos += M5.Lcd.drawChar('0', xpos, ysecs, 6);

M5.Lcd.drawNumber(ss, xpos, ysecs, 6); // Draw seconds

}

}

}

// function to extract number from compile string (don’t need to understand this for now)

| static uint8_t conv2d(const char* p) { |  |
| --- | --- |
| uint8_t v = 0; |  |
| if ('0' <= *p && *p <= '9') |  |
| v = *p - '0'; |  |
| return 10 * v + *++p - '0'; |  |
| } |  |

Task 3: Create stop watch:

The stop watch should the show hour, minute, second and 1/100th second, where button A is start, button B is stop and button C is reset.

0:01:23

Start

Reset

Stop

Task 4: Create Countdown timer:

Button A for adjusting minute, Button B for adjusting second, Button C is to start or stop timer. Button A and B  is working as following: each press will increase the value of minute or second by one, if the button A is pressed longer than 5 seconds the value  of minute become zero. If the button B is pressed longer than 5 seconds the value of second become zero. When the counter is running, pressing button A or B has no effect, you must press stop button first. Each press on Button C will toggle the status of the timer (start/stop). If you press Button C long enough (more than 5 seconds) the value of counter become 0:00. Once the time reach 0.00, it should give a beep or some musical tone until any button is pressed.

Section_____________date__________   Group No._________

Name____________________Student ID______________

Name____________________Student ID______________

Name____________________Student ID______________

Laboratory 4:Speaker, Digital alarm clock

Task1: Speaker, Jingle Bell

Graded by _____________________Time________________

Task2: Basic digital clock

Graded by _____________________Time________________

Task 3: Stop watch

Graded by _____________________Time________________

Task 4: Count down timer

Graded by _____________________Time________________